# Short Term Leave the Boat Checklist

Depending on how long we’ll leave the boat, some of these items may be omitted.

CommuterCruiser.com

______	Turn on dehumidifier.

______	Empty anything that might spoil out of fridge/cooler

______	Remove any food that might spoil in nets, hanging baskets or lockers

______	Close all seacocks

______	Leave key and contact information with marina or cruising friend

______	Turn off propane at tank and/or disconnect propane

______	Perform monthly battery check & top off water if needed

______	Clean boat – inside and out – so at least it’s clean when we return

______	Laundry – launder sheets & towels

______	Empty holding tank, flush with fresh water & empty again

______	Do the recommended storage technique for your head. Our VacuFlush has the electricity turned off, the seal broken, then cleaned and covered with saran wrap to making a tight seal keeping air out – we leave a bit of water in the head.

______	Put Raritan CP (cleans potties) in all closed drains

______	Check bilge and clean if necessary.

______	Tie dinghy down.

______	Lock any valuables left on deck, tie down everything else that might move.

______	Leave power off on electrical panel to everything except dehumidifier, battery charger, refrigerator and bilge

______	Double check all lines, add chafing gear if necessary.

______	Top off water tanks & add tablespoon bleach

______	Store all removable cockpit electronics &

______	Cover all remaining cockpit electronics

______	Store upper deck cushions in locker.

______	Teak back deck furniture folded & stored in main salon.

______	American Flag inside

______	Camping propane canister removed from grill & stored or discarded

______	Close and secure all hatches and portholes

______	Doors securely locked.